# Mqtt_Laboratory

Today you will have 2 papers to work on in this laboratory, so let's get started

What Is MQTT?

Message queuing telemetry support (MQTT) is defined as a low bandwidth consumption machine-to-machine protocol that helps IoT devices communicate with each other, with minimal code requirements and network footprint.

source:https://www.spiceworks.com/tech/iot/articles/what-is-mqtt/

# TASK 1- Connection interface Python and Robot IRB1600

Video:https://youtu.be/E-Gvn6QCAYc

# Code: Mqtt_Socket

import socket

import sys

import random

import paho.mqtt.client as mqtt

broker = 'broker.emqx.io'

port = 1883

topic = "button_topic/mqtt"

client_id = f'python-mqtt-{random.randint(0, 100)}'

def on_connect(client, userdata, flags, rc):

if rc == 0:

print("Connected to MQTT Broker!")

else:

print("Failed to connect, return code %d\n", rc)

def on_message(client, userdata, msg):

print(f"Received `{msg.payload.decode()}` from `{msg.topic}` topic")

trimitere_primire_mesaje(msg.payload.decode())

def connect_mqtt() -> mqtt.Client:

client = mqtt.Client(client_id)

client.on_connect = on_connect

client.on_message = on_message

client.connect(broker, port)

client.subscribe(topic)

return client

def trimitere_primire_mesaje(topiccc):

adresa_serverului = ("192.168.125.1", 1500)

print(sys.stderr, "Conectare la adresa %s si portul %s" % adresa_serverului)

priza_ethernet = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

priza_ethernet.connect(adresa_serverului)

print("Ca sa actionezi robotul ai de ales intre doua aplicatii")

print("Tasteaza aplicatia_1 sau aplicatia_2")

print("Orice alt mesaj va duce robotul in pozitia de parcare")

mesajul_introdus = topiccc.encode()

if mesajul_introdus == b"1":

print("Ai ales prima aplicatie1")

a = b"aplicatia_1"

elif mesajul_introdus == b"2":

print("Ai ales a doua aplicatie2")

a = b"aplicatia_2"

else:

print("Ai ales sa parchezi robotul")

a = b"aplica"

print(sys.stderr, "Se va trimite mesajul %s" % a)

priza_ethernet.sendall(a)

mesaj_primit = repr(priza_ethernet.recv(1025))

print("Mesajul primit este: %s" % mesaj_primit)

priza_ethernet.close()

if __name__ == "__main__":

client = connect_mqtt()

try:

client.loop_forever()

except KeyboardInterrupt:

client.disconnect()

client.loop_stop()

# Explained

This code establishes a connection to an MQTT broker and listens for messages on a specified topic, interacting with a robot through a TCP socket based on the received messages. Here’s a breakdown of the code:

### 1. Imports

python

Copy code

import socket

import sys

import random

import paho.mqtt.client as mqtt

- socket: Used for networking, specifically to connect to a TCP server.

- sys: Provides access to some system-related functions like stderr for error output.

- random: Generates random numbers (used here for generating a unique MQTT client ID).

- paho.mqtt.client: A Python library for implementing MQTT clients for communication with MQTT brokers.

### 2. MQTT Broker and Topic Setup

python

Copy code

broker = 'broker.emqx.io'

port = 1883

topic = "button_topic/mqtt"

client_id = f'python-mqtt-{random.randint(0, 100)}'

- broker: Specifies the MQTT broker address (broker.emqx.io is a free public MQTT broker).

- port: MQTT typically uses port 1883.

- topic: The topic on which this client will subscribe to receive messages.

- client_id: A unique client ID is generated using random.randint(0, 100) to avoid conflicts.

### 3. Callback Functions

These functions are called when specific MQTT events happen.

python

Copy code

def on_connect(client, userdata, flags, rc):

if rc == 0:

print("Connected to MQTT Broker!")

else:

print("Failed to connect, return code %d\n", rc)

- on_connect: Triggered when the client successfully connects to the broker. If the connection is successful (rc == 0), it prints a success message. Otherwise, it prints an error message.

python

Copy code

def on_message(client, userdata, msg):

print(f"Received `{msg.payload.decode()}` from `{msg.topic}` topic")

trimitere_primire_mesaje(msg.payload.decode())

- on_message: Triggered when a message is received from the subscribed topic. It decodes the message and calls the function trimitere_primire_mesaje to handle it.

### 4. MQTT Connection and Subscription

python

Copy code

def connect_mqtt() -> mqtt.Client:

client = mqtt.Client(client_id)

client.on_connect = on_connect

client.on_message = on_message

client.connect(broker, port)

client.subscribe(topic)

return client

- connect_mqtt: Creates a new MQTT client, assigns the callback functions for connection and message events, connects to the broker, and subscribes to the specified topic.

### 5. Handling Messages with a TCP Socket

python

Copy code

def trimitere_primire_mesaje(topiccc):

adresa_serverului = ("192.168.125.1", 1500)

print(sys.stderr, "Conectare la adresa %s si portul %s" % adresa_serverului)

priza_ethernet = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

priza_ethernet.connect(adresa_serverului)

print("Ca sa actionezi robotul ai de ales intre doua aplicatii")

print("Tasteaza aplicatia_1 sau aplicatia_2")

print("Orice alt mesaj va duce robotul in pozitia de parcare")

- trimitere_primire_mesaje: Handles the message received via MQTT. It sets up a TCP connection to a server with the IP 192.168.125.1 on port 1500.

  - The message is interpreted as a command to control a robot (based on whether it’s 1, 2, or something else).

  - Depending on the message, it sends commands like aplicatia_1 or aplicatia_2 to the server to control the robot.

  - If the message is not 1 or 2, it sends a message to park the robot.

The code then sends the selected command through the socket using:

python

Copy code

priza_ethernet.sendall(a)

It waits for a response from the server and prints the received message:

python

Copy code

mesaj_primit = repr(priza_ethernet.recv(1025))

print("Mesajul primit este: %s" % mesaj_primit)

Finally, the connection to the server is closed:

python

Copy code

priza_ethernet.close()

### 6. Main Loop

python

Copy code

if __name__ == "__main__":

client = connect_mqtt()

try:

client.loop_forever()

except KeyboardInterrupt:

client.disconnect()

client.loop_stop()

- Main loop: The MQTT client connects to the broker and enters an infinite loop to listen for incoming messages (client.loop_forever()).

- When you stop the program (e.g., via a keyboard interrupt), it disconnects from the broker gracefully.

### Summary

- The script connects to an MQTT broker and listens to messages on the topic button_topic/mqtt.

- When a message is received, it’s processed and sent over a TCP connection to a robot control server.

- Based on the message received, the robot is instructed to perform one of two tasks or park itself.

# Publisher Mqtt_Interface

import tkinter as tk

from PIL import Image, ImageTk

import paho.mqtt.client as mqtt

import random

# MQTT broker settings

broker = 'broker.emqx.io'

port = 1883

topic = "button_topic/mqtt"

# generate client ID with pub prefix randomly

client_id = f'python-mqtt-{random.randint(0, 1000)}'

# username = 'emqx'

# password = 'public'

def button_pressed(number):

mqtt_client.publish(topic, number)

print(f"Button {number} pressed and sent to MQTT broker")

def on_connect(client, userdata, flags, rc):

if rc == 0:

connection_label.config(text="Connected", fg="green")

else:

connection_label.config(text="Not Connected", fg="red")

# Create the main window

window = tk.Tk()

window.title("Button Interface")

# Create a title label

title_label = tk.Label(window, text="Button Interface", font=("Arial", 16, "bold"))

title_label.pack(pady=20)

# List of button labels and image file paths

button_data = [

{"label": "Button 1", "image_path": r"C:/Users/PCRoboti/Downloads/Proiect_conectare_MQTT_with_cloud/Proiect_conectare_MQTT_with_cloud/Picture/Figure1.jpg"},

{"label": "Button 2", "image_path": r"C:/Users/PCRoboti/Downloads/Proiect_conectare_MQTT_with_cloud/Proiect_conectare_MQTT_with_cloud/Picture/Figure2.jpg"},

{"label": "Button 3", "image_path": r"C:/Users/PCRoboti/Downloads/Proiect_conectare_MQTT_with_cloud/Proiect_conectare_MQTT_with_cloud/Picture/Figure3.jpg"}

]

# Create a frame to hold the buttons

button_frame = tk.Frame(window)

button_frame.pack()

# Configure grid weight for resizing

button_frame.grid_columnconfigure(0, weight=1)

button_frame.grid_columnconfigure(1, weight=1)

button_frame.grid_columnconfigure(2, weight=1)

# Create the MQTT client

mqtt_client = mqtt.Client()

# Set username and password for MQTT broker

#mqtt_client.username_pw_set(username, password=password)

# Assign the on_connect callback function

mqtt_client.on_connect = on_connect

# Connect to the MQTT broker

#mqtt_client.connect(broker, port=port)

mqtt_client.connect(broker, port)

# Create the buttons dynamically

for index, data in enumerate(button_data):

label = data["label"]

image_path = data["image_path"]

# Load the image and resize it

image = Image.open(image_path)

image = image.resize((350, 350))  # Adjust the dimensions as needed

# Create the photo image object

photo = ImageTk.PhotoImage(image)

# Create the button with image and label

button = tk.Button(button_frame, text=label, image=photo, compound=tk.TOP,

command=lambda num=index+1: button_pressed(num))

button.image = photo  # Store a reference to the image to prevent garbage collection

# Place the button in the frame

button.grid(row=0, column=index, padx=10, pady=10, sticky="nsew")

# Create a label to indicate the connection status

connection_label = tk.Label(window, text="Not Connected", fg="red")

connection_label.pack()

# Start the MQTT loop

mqtt_client.loop_start()

# Start the GUI event loop

window.mainloop()

# Disconnect from the MQTT broker

mqtt_client.disconnect()

# Explained

This code creates a graphical user interface (GUI) using Tkinter that displays a set of buttons with images. When a button is clicked, a message is sent to an MQTT broker using the paho-mqtt library. The code includes the following components:

### 1. Imports

python

Copy code

import tkinter as tk

from PIL import Image, ImageTk

import paho.mqtt.client as mqtt

import random

- tkinter: A built-in Python module for creating GUIs.

- PIL.Image and PIL.ImageTk: Part of the Python Imaging Library (Pillow), used for handling image files and displaying them in the GUI.

- paho.mqtt.client: A library for interacting with an MQTT broker.

- random: Used to generate a unique MQTT client ID.

### 2. MQTT Broker Settings

python

Copy code

broker = 'broker.emqx.io'

port = 1883

topic = "button_topic/mqtt"

client_id = f'python-mqtt-{random.randint(0, 1000)}'

- broker: The MQTT broker address (broker.emqx.io is a free broker).

- port: Standard MQTT port (1883).

- topic: The topic to which button-press messages will be sent.

- client_id: A unique ID for the MQTT client to avoid conflicts.

### 3. Button Press Handler

python

Copy code

def button_pressed(number):

mqtt_client.publish(topic, number)

print(f"Button {number} pressed and sent to MQTT broker")

- button_pressed(number): Publishes a message to the MQTT broker whenever a button is pressed. The message contains the button number.

### 4. MQTT Connection Callback

python

Copy code

def on_connect(client, userdata, flags, rc):

if rc == 0:

connection_label.config(text="Connected", fg="green")

else:

connection_label.config(text="Not Connected", fg="red")

- on_connect(client, userdata, flags, rc): A callback that runs when the MQTT client connects to the broker. If the connection is successful (rc == 0), it updates the label in the GUI to show "Connected." Otherwise, it shows "Not Connected."

### 5. Tkinter GUI Setup

Main Window:
python
Copy code
window = tk.Tk()

window.title("Button Interface")

- Creates the main window for the GUI.

Title Label:
python
Copy code
title_label = tk.Label(window, text="Button Interface", font=("Arial", 16, "bold"))

title_label.pack(pady=20)

- Adds a title at the top of the window.

Button Data:
python
Copy code
button_data = [

{"label": "Button 1", "image_path": r"path_to_image1"},

{"label": "Button 2", "image_path": r"path_to_image2"},

{"label": "Button 3", "image_path": r"path_to_image3"}

]

- This is a list of dictionaries where each entry contains:

  - label: The button's label.

  - image_path: The path to the image associated with each button.

Button Frame:
python
Copy code
button_frame = tk.Frame(window)

button_frame.pack()

- Creates a frame to hold the buttons.

Grid Configuration:
python
Copy code
button_frame.grid_columnconfigure(0, weight=1)

button_frame.grid_columnconfigure(1, weight=1)

button_frame.grid_columnconfigure(2, weight=1)

- Sets up a grid for the buttons, allowing the frame to resize the buttons equally.

### 6. MQTT Client Setup

python

Copy code

mqtt_client = mqtt.Client()

mqtt_client.on_connect = on_connect

mqtt_client.connect(broker, port)

- mqtt_client = mqtt.Client(): Initializes the MQTT client.

- mqtt_client.on_connect = on_connect: Assigns the on_connect callback to handle MQTT connection events.

- mqtt_client.connect(broker, port): Connects to the MQTT broker.

### 7. Creating Buttons with Images

python

Copy code

for index, data in enumerate(button_data):

label = data["label"]

image_path = data["image_path"]

image = Image.open(image_path)

image = image.resize((350, 350))  # Resize image

photo = ImageTk.PhotoImage(image)

button = tk.Button(button_frame, text=label, image=photo, compound=tk.TOP,

command=lambda num=index+1: button_pressed(num))

button.image = photo  # Prevents the image from being garbage collected

button.grid(row=0, column=index, padx=10, pady=10, sticky="nsew")

- enumerate(button_data): Loops through each button's data (label and image path).

- Image Handling: Loads the image from the path and resizes it to fit within the GUI.

- Button Creation: A button is created for each entry in button_data, displaying the image and text. The command argument binds the button press to the button_pressed function, passing the button's index as the argument.

### 8. Connection Status Label

python

Copy code

connection_label = tk.Label(window, text="Not Connected", fg="red")

connection_label.pack()

A label is added to the window to display the connection status of the MQTT client.

### 9. Starting MQTT and GUI Loops

python

Copy code

mqtt_client.loop_start()

window.mainloop()

- mqtt_client.loop_start(): Starts the MQTT client loop in a background thread. This is necessary for handling incoming MQTT messages and maintaining the connection.

- window.mainloop(): Starts the Tkinter event loop, keeping the GUI open.

### 10. Disconnecting from the Broker

python

Copy code

mqtt_client.disconnect()

This disconnects the MQTT client when the GUI is closed.

### Summary

- The script creates a GUI with buttons that each send a message to an MQTT broker when pressed.

- Images are loaded and displayed on each button.

- The MQTT connection status is displayed in the GUI, and the MQTT client runs in the background while the GUI is active.

# TASK 2 - Connection between phone and ESP8266

# Phone configuration

# Subscriber Mqtt

#include <ESP8266WiFi.h>  // For ESP8266 boards

#include <PubSubClient.h>

#include <ArduinoJson.h>  // For parsing JSON messages

const char* ssid = "BogdanS24";  // Your Wi-Fi network

const char* password = "12345678";  // Wi-Fi password

const char* mqtt_server = "broker.emqx.io";

const int mqtt_port = 1883;  // Use 1883 for non-secure connections

const char* topic = "xarm/variables";  // The topic to subscribe to

WiFiClient net;

PubSubClient client(net);

unsigned long previousMillis = 0; // To track time for blinking

unsigned long blinkInterval = 0;   // Interval for blinking the LED

int blinkCount = 0;  // To track how many times to blink

bool ledState = LOW;

bool isBlinking = false;  // Flag to track if blinking is happening

// Message buffer (simple queue)

#define MAX_QUEUE_SIZE 5

struct Message {

String variable1;

int variable2;

};

Message messageQueue[MAX_QUEUE_SIZE];  // Buffer to store incoming messages

int queueStart = 0, queueEnd = 0;      // Pointers to manage the queue

void messageReceived(char* topic, byte* payload, unsigned int length) {

StaticJsonDocument<200> doc;

DeserializationError error = deserializeJson(doc, payload, length);

if (error) {

Serial.print("deserializeJson() failed: ");

Serial.println(error.c_str());

return;

}

String variable1 = doc["variable1"];  // Get 'a' or 'r'

int variable2 = doc["variable2"];  // Get number of blinks

Serial.println("Message received:");

Serial.print("variable1: ");

Serial.println(variable1);

Serial.print("variable2: ");

Serial.println(variable2);

// Add the message to the queue if space is available

if ((queueEnd + 1) % MAX_QUEUE_SIZE != queueStart) {

messageQueue[queueEnd].variable1 = variable1;

messageQueue[queueEnd].variable2 = variable2;

queueEnd = (queueEnd + 1) % MAX_QUEUE_SIZE;

} else {

Serial.println("Message queue is full! Ignoring new message.");

}

}

void processMessageQueue() {

// If not currently blinking and the queue has messages, process the next one

if (!isBlinking && queueStart != queueEnd) {

Message msg = messageQueue[queueStart];

queueStart = (queueStart + 1) % MAX_QUEUE_SIZE;

// Set the blink interval based on variable1

if (msg.variable1 == "a") {

blinkInterval = 1000;  // 1 second for 'a'

} else if (msg.variable1 == "r") {

blinkInterval = 2000;  // 2 seconds for 'r'

}

blinkCount = msg.variable2;  // Set the number of blinks

isBlinking = true;  // Set the flag to indicate that blinking is in progress

}

}

void setup() {

Serial.begin(115200);

pinMode(LED_BUILTIN, OUTPUT);  // Onboard LED as output

// Connect to Wi-Fi

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting to WiFi...");

}

Serial.println("Connected to WiFi");

// Configure MQTT client

client.setServer(mqtt_server, mqtt_port);

client.setCallback(messageReceived);

// Connect to the MQTT broker

reconnectMQTT();

client.subscribe(topic);  // Subscribe after connecting

}

void loop() {

if (!client.connected()) {

reconnectMQTT();

}

client.loop();

// Process the next message from the queue if blinking is not in progress

processMessageQueue();

// Handle the blinking process

if (isBlinking) {

unsigned long currentMillis = millis();

if (currentMillis - previousMillis >= blinkInterval) {

previousMillis = currentMillis;

// Toggle LED state

ledState = !ledState;

digitalWrite(LED_BUILTIN, ledState);

// If LED is off, reduce the blink count (we count one complete cycle)

if (ledState == LOW) {

blinkCount--;

}

// If the blink count reaches 0, stop blinking

if (blinkCount <= 0) {

isBlinking = false;

Serial.println("Blinking finished.");

}

}

}

}

void reconnectMQTT() {

while (!client.connected()) {

Serial.println("Connecting to MQTT...");

if (client.connect("arduinoClient")) {

Serial.println("Connected to MQTT");

client.subscribe(topic);  // Re-subscribe after connecting

} else {

Serial.print("Failed to connect, rc=");

Serial.print(client.state());

Serial.println(" Trying again in 5 seconds...");

delay(5000);

}

}

}

# Explained

This code is designed to run on an ESP8266 microcontroller, which connects to an MQTT broker and handles JSON messages received over the MQTT protocol. The messages instruct the device to blink an onboard LED with specific intervals and durations. Here’s a breakdown of the code:

### 1. Includes and Constants

cpp

Copy code

#include <ESP8266WiFi.h>  // For ESP8266 boards

#include <PubSubClient.h>  // For MQTT functionality

#include <ArduinoJson.h>   // For parsing JSON messages

- ESP8266WiFi.h: Allows the ESP8266 to connect to Wi-Fi.

- PubSubClient.h: A library for MQTT client functionality.

- ArduinoJson.h: Used to deserialize JSON-formatted payloads.

cpp

Copy code

const char* ssid = "BogdanS24";  // Your Wi-Fi network

const char* password = "12345678";  // Wi-Fi password

const char* mqtt_server = "broker.emqx.io";

const int mqtt_port = 1883;

const char* topic = "xarm/variables";  // The topic to subscribe to

- Wi-Fi credentials: ssid and password represent the Wi-Fi network details.

- mqtt_server, mqtt_port: The MQTT broker address and port for non-secure connections (port 1883).

- topic: The MQTT topic on which messages are received.

### 2. Wi-Fi and MQTT Setup

cpp

Copy code

WiFiClient net;

PubSubClient client(net);

- WiFiClient net: Used to manage the Wi-Fi connection.

- PubSubClient client: Handles the MQTT communication.

### 3. Message Queue and Blinking Variables

cpp

Copy code

unsigned long previousMillis = 0;  // Tracks time for blinking

unsigned long blinkInterval = 0;   // Interval for blinking the LED

int blinkCount = 0;                // Number of blinks

bool ledState = LOW;               // LED state (on or off)

bool isBlinking = false;           // Flag to indicate if blinking is happening

- These variables track the current state of the LED (on or off), manage blinking intervals, and control how many times the LED should blink.

cpp

Copy code

#define MAX_QUEUE_SIZE 5

struct Message {

String variable1;

int variable2;

};

Message messageQueue[MAX_QUEUE_SIZE];

int queueStart = 0, queueEnd = 0;

- Message Queue: A simple queue to store up to 5 messages received from the MQTT broker. Each message contains two variables: variable1 (string) and variable2 (integer).

### 4. Handling Incoming MQTT Messages

cpp

Copy code

void messageReceived(char* topic, byte* payload, unsigned int length) {

StaticJsonDocument<200> doc;

DeserializationError error = deserializeJson(doc, payload, length);

if (error) {

Serial.print("deserializeJson() failed: ");

Serial.println(error.c_str());

return;

}

String variable1 = doc["variable1"];

int variable2 = doc["variable2"];

- messageReceived: Callback function that is invoked whenever a new MQTT message is received. It processes the JSON payload, extracting two values:

  - variable1: Controls the blinking interval ("a" or "r").

  - variable2: The number of times the LED should blink.

cpp

Copy code

if ((queueEnd + 1) % MAX_QUEUE_SIZE != queueStart) {

messageQueue[queueEnd].variable1 = variable1;

messageQueue[queueEnd].variable2 = variable2;

queueEnd = (queueEnd + 1) % MAX_QUEUE_SIZE;

} else {

Serial.println("Message queue is full! Ignoring new message.");

}

- Adds the message to the queue if there's space. The queue works in a circular buffer style.

### 5. Processing the Message Queue

cpp

Copy code

void processMessageQueue() {

if (!isBlinking && queueStart != queueEnd) {

Message msg = messageQueue[queueStart];

queueStart = (queueStart + 1) % MAX_QUEUE_SIZE;

if (msg.variable1 == "a") {

blinkInterval = 1000;

} else if (msg.variable1 == "r") {

blinkInterval = 2000;

}

blinkCount = msg.variable2;

isBlinking = true;

}

}

- processMessageQueue(): Dequeues messages from the queue and configures the blink interval and the number of blinks. It checks if blinking is not already in progress.

### 6. Setup Function

cpp

Copy code

void setup() {

Serial.begin(115200);

pinMode(LED_BUILTIN, OUTPUT);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting to WiFi...");

}

Serial.println("Connected to WiFi");

client.setServer(mqtt_server, mqtt_port);

client.setCallback(messageReceived);

reconnectMQTT();

client.subscribe(topic);

}

- setup(): Initializes the ESP8266:

  - Connects to Wi-Fi.

  - Sets up the MQTT client and its callback function (messageReceived).

  - Subscribes to the MQTT topic after connecting to the broker.

### 7. Main Loop

cpp

Copy code

void loop() {

if (!client.connected()) {

reconnectMQTT();

}

client.loop();

processMessageQueue();

if (isBlinking) {

unsigned long currentMillis = millis();

if (currentMillis - previousMillis >= blinkInterval) {

previousMillis = currentMillis;

ledState = !ledState;

digitalWrite(LED_BUILTIN, ledState);

if (ledState == LOW) {

blinkCount--;

}

if (blinkCount <= 0) {

isBlinking = false;

Serial.println("Blinking finished.");

}

}

}

}

- loop():

  - Ensures the MQTT client stays connected and processes incoming messages.

  - If there's a message in the queue and blinking is not in progress, it starts the blinking sequence.

  - Controls the LED blinking based on the current time (millis()), toggling the LED state and reducing the blink count with each cycle.

### 8. Reconnect MQTT Function

cpp

Copy code

void reconnectMQTT() {

while (!client.connected()) {

Serial.println("Connecting to MQTT...");

if (client.connect("arduinoClient")) {

Serial.println("Connected to MQTT");

client.subscribe(topic);

} else {

Serial.print("Failed to connect, rc=");

Serial.print(client.state());

Serial.println(" Trying again in 5 seconds...");

delay(5000);

}

}

}

- reconnectMQTT(): Ensures the ESP8266 is connected to the MQTT broker. If the connection is lost, it attempts to reconnect every 5 seconds.

### Summary

- The ESP8266 connects to an MQTT broker and subscribes to a topic.

- JSON-formatted MQTT messages control the blinking behavior of the onboard LED.

- The message specifies:

  - variable1: Determines the blink interval ("a" for 1 second, "r" for 2 seconds).

  - variable2: The number of times the LED should blink.

- Messages are processed in a queue to handle multiple commands sequentially.